Real-world Image Enhancement and Analysis

## 1. Introduction (The Problem)

Detecting foreign objects, such as bullets, in X-ray images is often challenging because the object may be small and hidden within low-contrast regions of the body. Enhancing the X-ray improves visibility of such objects, making medical diagnosis easier and more reliable.

## 2. Dataset

- Source: X-ray image collected from Google Images.

- Format: PNG.

- Resolution: 614 × 630 pixels.

- Reason for Selection: The image contained a small metallic object (bullet) that was faint and difficult to detect due to limited contrast.

## 3. Methodology & Justification

### 3.1 Negative Transformation

- Formula: s=255−r

- Justification: Inverts intensity values so that bright areas become dark and dark areas become bright. This highlighted the bullet, which appeared much darker than the surrounding tissue.

### 3.2 Logarithmic Transformation

- Formula: s=c⋅log(1+r)

- Justification: Expands darker pixel values and compresses brighter regions. This makes hidden details in low-intensity areas more visible, useful for medical images with shadows.

### 3.3 Gamma (Power-Law) Transformation

- Formula: s=c⋅r^γ

- Justification: By adjusting γ\gammaγ, overall brightness and contrast were controlled.

  - γ<1: Brightened shadows, making the bullet more visible.

  - γ>1: Darkened bright areas.

### 3.4 Thresholding

- Formula:

- Justification: Segmented the image into black and white. However, because the bullet was very small, this method did not yield clear results.

### 3.5 Contrast Stretching (Piecewise Linear)

- Formula:

- Justification: Enhanced the global contrast of the image, though results were only moderate.

### 3.6 Histogram Equalization

- Formula:

- Justification: Redistributes pixel intensities to improve overall contrast. The bullet became easier to identify due to global brightness adjustment.

### 3.7 Combination: Logarithmic + Contrast Stretching

- Step 1: Logarithmic Transform s1=c⋅log(1+r).

- Step 2: Contrast Stretching on s1​.

- Justification: The log transform enhanced low-intensity details, while contrast stretching boosted overall contrast. This combination gave strong results but required parameter fine-tuning.

## 4. Results & Analysis

### 4.1 Negative Transformation

- Observation: Bullet appeared much clearer as it contrasted sharply against lighter tissues.

### 4.2 Logarithmic Transformation

- Observation: Helped reveal shadow regions but slightly compressed brighter areas.

### 4.3 Gamma Transformation

- Observation: With γ<1, bullet visibility improved greatly. With γ>1\, results worsened.

### Thresholding

- Observation: Failed to isolate the bullet, as small object details were lost.

### 4.5 Contrast Stretching

- Observation: Moderate improvement, but not as effective as Negative or Gamma.

### 4.6 Histogram Equalization

- Observation: Improved global contrast and made the bullet easier to detect, though sometimes over-enhanced surrounding tissues.

### 4.7 Log + Contrast Combo

- Observation: Produced one of the best results by enhancing shadows and adjusting global contrast.

### 5. Critical Evaluation

- Most Effective Techniques: Negative and Gamma Transformations clearly highlighted the bullet.

- Least Effective: Thresholding, as it oversimplified the image and lost fine details.

- Combination: Log + Contrast Stretching produced strong results but required careful parameter selection.

### 6. Conclusion

Among all tested techniques, Negative and Gamma transformations were most effective in detecting small foreign objects such as bullets in X-ray images. Histogram Equalization and the Log+Contrast combo also provided significant improvements. Thresholding was not suitable in this context. Overall, image enhancement can greatly support medical diagnostics by making critical details more visible.